# Quantum-Assisted UAV Routing for Post Disaster Relief — Technical Workflow

Author: Harshita Bothra (B.Tech Robotics & Automation, SIT Pune)
Date: September 6, 2025
Inspired by: QNu Labs (India), QpiAI (India), Quantinuum, QC Ware

SUMMARIZATION

Rapid post‑disaster response hinges on routing small UAVs (quadcopters/VTOLs) to deliver medical kits and sensors under tight time windows, battery/payload limits, and dynamic hazards. This document proposes a hybrid quantum–classical workflow that casts a constrained Vehicle Routing Problem with Time Windows (VRPTW) into a QUBO and solves subproblems using QAOA on simulators and gate‑based QPUs (pilot‑scale), then stitches/repairs solutions with classical metaheuristics. The pipeline also integrates quantum‑safe communications (QKD/QRNG) for command‑and‑control. We outline algorithmic choices, tech stack, evaluation, and risks.

# PROBLEM DEFINITION & SCOPE

Goal: Minimize total mission time and missed‑delivery penalties while respecting payload and battery constraints, site time windows, and no‑fly zones. Inputs: depot location, demand points with GPS/time windows, UAV specs. Outputs: feasible multi‑UAV routes and KPIs. Quantum advantage: NP‑hard VRPTW may benefit from QAOA’s ability to explore large combinatorial spaces with subinstances solved faster than heuristics in some cases.

# WORKFLOW OVERVIEW

1. Data & preprocessing: cluster points, adjust travel times by wind.
2. QUBO modeling: encode routes and penalties.
3. Hybrid solver: QAOA with depth p=1..3, classical optimizer, compilation, and repair.
4. Global merge: stitch subinstances with Large Neighborhood Search.
5. Validation & rollout: simulate routes, check constraints, secure with QKD/QRNG.

# Mathematical Sketch (QUBO)

Binary variables x_ij encode edges. Objective: minimize travel cost + lateness + energy + capacity penalties. Constraints: exactly one incoming/outgoing edge per node, plus quadratic penalties for battery/time windows.

# Algorithm Design

standard QAOA with constraint‑preserving mixers. Initialization: warm‑start from heuristics or LP relaxation. Optimizer: SPSA/COBYLA. Compilation: pytket for depth reduction. Repair: classical 2‑opt and feasibility enforcement.

# Tech Stack & Rationale

- Classical baseline: Python, OR‑Tools, NetworkX.
- Quantum SDKs: IBM Qiskit, Google Cirq, NVIDIA CUDA‑Q, Quantinuum pytket.
- Cloud/Devices: IBM Quantum, Quantinuum H‑Series, Amazon Braket.
- Security: QNu Labs Tropos (QRNG) and Armos (QKD).

# Benefits vs Disadvantages

| Aspect | Quantum-assisted | Classical-only |
| --- | --- | --- |
| Solution quality | May escape local minima | Near-optimal but not global |
| Runtime | Limited scaling | Fast, scalable |
| Constraints | Encoded via penalties | Direct handling |
| Noise | Sensitive to hardware errors | Stable |

# Risks & Mitigations

- Qubit count/noise: keep subinstances small, use mitigation.
- Constraint handling: mixers and penalty calibration, classical repair.
- Operational uncertainty: rolling horizon planning and fast local repair.

# References

1. IBM Qiskit Docs – QAOA, Optimization module.
2. Google Cirq QAOA routing examples.
3. NVIDIA CUDA‑Q documentation.
4. Quantinuum H‑Series, pytket user guide.
5. Amazon Braket developer docs.
6. Nature Scientific Reports (2024) – VRP QUBO models.
7. D‑Wave Ocean SDK VRP examples.
8. QNu Labs product pages.
9. QpiAI news and ecosystem docs.
10. arXiv/tutorials on QAOA‑VRP (2024–25).

# Toy Example: UAV Routing as a QUBO

This document illustrates, in a more human-readable way, how a small-scale UAV routing problem can be mapped into a Quadratic Unconstrained Binary Optimization (QUBO) formulation. We use 3 UAVs, 5 delivery sites, and one depot.

## Problem Setup

Imagine a disaster relief scenario where 3 UAVs must deliver supplies to 5 different sites. Each UAV starts and ends at the depot (located at coordinate (0,0)). The sites are scattered with fixed coordinates. The task is to find routes for UAVs such that all sites are visited exactly once, without violating UAV constraints.

Coordinates used in this toy problem:

Site 0: (0.0, 0.0)
Site 1: (2.0, 1.0)
Site 2: (1.0, 3.0)
Site 3: (4.0, 4.0)
Site 4: (6.0, 1.0)
Site 5: (5.0, -1.0)

## Encoding into QUBO

We create binary variables x[k, site, t] that equal 1 if UAV k visits a particular site at position t in its route. For 3 UAVs, 5 sites, and 5 positions, this gives 75 binary variables.

The QUBO objective combines:
• Travel costs (distance traveled between consecutive visits)
• Penalties to ensure each site is visited exactly once
• Penalties to ensure each UAV visits at most one site per position

## Example Assignments

We evaluated two assignments of UAV routes:

✅ Feasible Example (all constraints respected):

• UAV1 → Site 1 → Site 2 → Depot
• UAV2 → Site 3 → Site 4 → Depot
• UAV3 → Site 5 → Depot
QUBO Energy = -881.166 (lower = better)

❌ Infeasible Example (Site 1 visited twice):

• UAV1 → Site 1 → Site 2 → Depot
• UAV2 → Site 1 → Site 3 → Site 4 → Depot
• UAV3 → Site 5 → Depot
QUBO Energy = -874.93 (penalized higher)

## Interpretation

The feasible route assignment produces a lower QUBO energy, while the infeasible assignment is penalized. This demonstrates how QUBO naturally encodes both the objective and the constraints. A quantum algorithm like QAOA would search for low-energy solutions to this QUBO, effectively suggesting optimal UAV routes.

## Supporting Files

Alongside this descriptive explanation, we also generated two CSV files:
• toy_var_map.csv – variable index mapping to UAV, site, and position
• toy_qubo_sparse.csv – the full QUBO sparse coefficient list